Man Who Came To Dinner

SOUND CHECKLISTS

PRESHOW

1) Power Up

A) Big UPS

B) Little UPS

C) Computer

D) Board

E) Playback Rack (CD PLAYER)

F) Clear Com

G) Amp Racks

2) Headset Batteries & Mics to Backstage

A) Door Buzzer at angle AWAY from Speaker

B) Foley Door Mic Inside Triangle

C) Front Door Mic No Longer Needed

4) While On Deck, Check ALL Cable Runs with Mag Lite

- From Mics to Snake

- Snake Connections Secure

- Floor Mic Runs Secure and Dressed

- Floor Mic Positions Verified

- Left/Radio Speaker, From Wall Jack and Back of Speaker Jack

- Right/Buzzer Speaker, From Wall Jack and Back of Speaker Jack

- Check Wiring Secure in Back of Piano Speaker in Windowseat

5) Take Old Batteries and Mic Case Upstairs.  Charge Batteries.

6) Check Signal of ALL Mics on Board.

7) MUTE ALL MIC CHANNELS UNTIL SHOW GO

8) Check CATWALK SPEAKER, Connections in Speaker and From Jack SECURE.  TIELINE SECURE AND DUAL SAFETIES IN PLACE.

9) Check Wireless Clear Com Powered ON and verify cable runs.

10) Check Piano Speaker Cable Drop SECURE.

11) ON BOARD – Check Speaker Connections with PINK NOISE

- Utilities, Oscillator, ON.

- Assign To Busses 1-8 INDIVIDUALLY

- Slowly Turn Dial Up Until You Hear Signal

- After ALL Speaker Checks, UNASSIGN OSCILLATOR AND TURN OFF

- Double Check Channel 5 EQ IS ON For Radio Effects.

12) Check Layer 2, Ch 17-18 ON, and Test Phone Ring Backup CD

13) Open SFX File (My Documents, SFX, MWCD.worksfx)

A) Double Check you are in PLAY Mode (Go Option Only, No Edit)

B) Verify Cues

C) Reset for Top of Show

TOP OF SHOW:

- Floor Mics ON

- All SFX Channels Soloed to Monitor Speakers, as well as anything else you want.

- Constant Standby for Practical Phone Ring Failure

INTERMISSIONS:

- ALL MICS MUTED

- CHANNEL 5 EQ PUNCHED OUT AFTER PENGUIN CUES

POSTSHOW:

- ALL MICS MUTED

- Collect Backstage Mics & Secure Floor Mics

- Wireless Headsets Secured in Organ Room

- Power Down

    - Amp Racks

    - Clear Com

    - Playback Rack

    - Board

    - Close SFX and Power Down Computer and Monitor

    - Switch OFF Little UPS

    - Big UPS OFF